# Tutorial (30/1) - Battery Datasets

### 1. The NASA Prognostics Center of Excellence (PCoE) Repository

The NASA PCoE dataset collection remains the seminal benchmark for battery prognostics. Established to support aerospace health management, these datasets provide "run-to-failure" trajectories that are rare in industrial datasets where testing is often truncated. The datasets typically feature commercially available 18650 Li-ion cells (eg, Gen 2 commercially available cells) cycled under strictly controlled thermal and electrical conditions. The protocols include standard Constant Current-Constant Voltage (CC-CV) charging followed by various discharge profiles, including constant impedance loads and, crucially, "Random Walk" operational profiles that mimic non-steady-state usage. The repository provides time-synchronous measurements of voltage, current, and temperature, alongside periodic reference discharge cycles used to measure the ground truth capacity. Electrochemical Impedance Spectroscopy (EIS) sweeps are also frequently provided, offering insights into the internal impedance evolution (Ohmic, Charge Transfer, and Diffusion resistances) as the cell ages.

Download the Datasets.[]

Download the data analysis[]

Suggested Applications : SOH, SOC, and RUL prediction

### 2. The CALCE Battery Research Group (University of Maryland)

The Center for Advanced Life Cycle Engineering (CALCE) provides a dataset that is complementary to NASA's, often focusing on different chemistries (eg, LiCoO2, LiFePO4) and form factors (prismatic and pouch cells). CALCE data is renowned for its focus on calendar aging (storage degradation) and dynamic stress testing. The datasets often include results from standard drive cycles such as the Dynamic Stress Test (DST) and the Federal Urban Driving Schedule (FUDS), which introduce rapid current fluctuations absent in constant-current datasets. Similar to NASA, CALCE provides cycle-by-cycle capacity degradation data. However, the CALCE datasets often explore a wider matrix of State of Charge (SOC) ranges and temperatures, allowing for the study of partial cycling effects—a critical factor in hybrid electric vehicle (HEV) applications.

Download the Datasets [ click hereLinks to an external site. ]

Suggested Application : SOH, SOC, and RUL prediction

### 3. Panasonic 18650PF Li-ion Battery Performance and Characterization Dataset

This dataset features experimental testing of a 2.9Ah Panasonic 18650PF cell . Using a Digatron Universal Battery Tester, the cell was subjected to a range of temperatures (25°C, 10°C, 0°C, -10°C, and -20°C) and dynamic drive cycles (US06, HWFET, UDDS, LA92) scaled to mimic the power demands of an electric Ford F150 . The collection provides high-fidelity data for characterizing lithium-ion performance under diverse environmental and load conditions .

Download the datasets [ click hereDownload click hereOpen this document with ReadSpeaker docReader ]

Suggested application: SOC prediction

### 4. NASA PCoE Randomized Battery Usage Dataset: Cells RW9–RW12

This dataset, identifying cells RW9, RW10, RW11, and RW12 , is a specialized component of the NASA Prognostics Center of Excellence (PCoE) Randomized Battery Usage Data Set . It records the aging of 18650 Li-ion batteries under a Random Walk (RW) operational profile, where charging and discharging currents fluctuate randomly between -4.5A and 4.5A . Current levels are updated every 5 minutes , creating a high-variance environment that more accurately reflects real-world usage than standard constant-current tests.

To monitor degradation, the random walk is paused every 1,500 periods (roughly every 5 days) to perform standardized reference charging and discharging cycles , providing clear benchmarks for the battery's health over time.

Download the datasets [ click hereLinks to an external site. ]

Suggested application: Dynamic SOC estimation, RUL prediction 


Other dataset links

- https://www.kaggle.com/datasets/ziya07/li-s-battery-thermal-failure-datasetLinks to an external site.

- https://kilthub.cmu.edu/articles/dataset/eVTOL_Battery_Dataset/14226830Links to an external site.

- https://ieee-dataport.org/documents/time-series-data-battery-energy-storage-systemLinks to an external site.

- https://zenodo.org/records/7244939Links to an external site.

- https://www.nrel.gov/transportation/battery-failureLinks to an external site.

- https://data.nasa.gov/dataset/randomized-and-recommissioned-battery-dataset/resource/cdea0a46-2c8d-4192-bbe7-8fbebfcfdb31Links to an external site.Links to an external site.